COMPUTER COMMUNICATION AND NETWORKING

( CCN )

PROJECT

TOPIC: GNS3 BASED VLAN ACL

Instructor: Sir Dileep Kumar

Pre-requisites of this project:

- Installation of GNS3

- VMware workstation

- C7200 router IOS Image

- Number of switches including c3640, c3725, layer2 switch.

At the start of this project, we downloaded the gns3 from www.gns3.com .

It is downloaded on a low spec laptop due to which I faced the overheating issues number of times.

The VMware workstation was also download from the above website.

Understanding that which switch would be the best to use in this project was too confusing as many switches are available on google.

We were sitting idle for almost 15 to 20 days as we were trying to understand the basic concept of VLAN and ACL and how they both combine to form a network. We watched many tutorial videos in which they were creating a VLAN but almost every video had a different approach so we picked the approach that was easy to understand.

Understanding VLAN:
Virtual Local Area Networks is a custom network which is created from one or more than one LAN. It enables a group of devices available in multiple networks to be combined into one logical network. Without VLANs, a broadcast sent from a host can easily reach all network devices. But by using VLANS, a broadcast can only reach the devices that are available in the same VLAN.

Understanding ACL:
ACL also known as Access Control List is basically a set of rules that determines that which packet should be accepted or rejected by which network. It performs packet filtering to control the movement of packets through a network. It provides security by limiting the access of traffic into a network.

HOW TO DOWNLOAD A ROUTER/SWITCH

- First, we have to download an IOS image of the router from any website.

I used https://protechgurus.com/download-gns3-ios-images/ this website for downloading Purpose.

- Then we will go to the Edit option from the top bar and click on ‘preference’. The above window will appear after clicking on it. Next click on the IOS routers option

- Now, select the New option and select the directory where IOS Image of the router is placed.

- After selecting the image, just follow the simple instructions that will download the router for you.

TOPOLOGY

Now, coming on to the main topic, there were number of ways that the topology of this project could be created. I tried many different topologies but due to lack of skills in networking area, I was facing problems in almost every topology. And in the last few days, this topology was decided by us:

I had the option to connect router with both the switches and to remove the direct link between them but that would be an ordinary approach. Therefore, I decided to implement the VLAN by connecting the two switches. I used the TRUNK link to connect the two switches. Trunk links are used to pass the VLAN information in between the switches. Moreover, I created two VLANs here VLAN 10 and VLAN 20, both have two Virtual PCs connected to them on different switches.

CONFIGURATION

I will try to cover the main parts of configuration of this project here and the screenshots of the configuration part for verification purpose are attached in the next section:

- At the start of this project, I started all nodes and changed the hostnames of switches by going in config mode

Switch1 > en

Switch1 # config

Switch1 (config) # hostname SW1

- After this, I created the switch 2 as VTP server and switch 1 as VTP client by using the command:

SW2 (config)# vtp mode server

SW1 (Config) # vtp mode client

- After this, Vlans were created by using the following commands

SW2 (Config)# Vlan 10

SW2(config-vlan)#vlan 20

- After this, trunk links have to be enabled and for this purpose, I used this command on both the ethernets connecting the two switches together:

SW2/SW1 (config-if)# switchport trunk encapsulation dotiq

SW2/SW1 (config-if)# Switchport mode trunk

- We check the Vlans and their names by using the command

SW2(Config)# do show vlan

- Next step is to assign IP addresses to the VLAN Interface

SW2 (config-if)# interface vlan 10

SW2 (config-if)# ip add 192.168.10.254 255.255.255.0

SW2 (config-if)# no shutdown

SW2 (config-if)#interface vlan 20

SW2 (config-if)#ip add 192.168.20.254 255.255.255.0

SW2 (config-if)#no shutdown

We can check the interface and ip assigned to them by using the command

SW2 (config-if)#do show ip interface brief

- Now, we have to assign the Vlans to the ethernet connection that connects it to the Virtual PC.

SW1 (config-if)# interface gig 0/1

SW1 (config-if)#switchport mode access

SW1 (config-if)#switchport access vlan 20

SW1 (config-if)#no shutdown

SW1 (config-if)# interface gig 0/2

SW1 (config-if)#switchport mode access

SW1 (config-if)#switchport access vlan 10

SW1 (config-if)#no shutdown

SW2 (config-if)# interface gig 0/1

SW2 (config-if)#switchport mode access

SW2 (config-if)#switchport access vlan 20

SW2 (config-if)#no shutdown

SW2 (config-if)# interface gig 0/2

SW2 (config-if)#switchport mode access

SW2 (config-if)#switchport access vlan 10

SW2 (config-if)#no shutdown

- After this, we will assign IP address to the Virtual PC.

PC1> ip 192.168.20.5 255.255.255.0 gateway 192.168.20.254

PC2> ip 192.168.10.5 255.255.255.0 gateway 192.168.10.254

PC3> ip 192.168.20.7 255.255.255.0 gateway 192.168.20.254

PC4> ip 192.168.10.7 255.255.255.0 gateway 192.168.10.254

- After assigning the Ips to the Virtual PC, now we have to create ACL.

I used the extended access control list here and the configuration is:

SW2 (config)# IP access-list extended CCN

SW2 (Config-ext-nacl)# deny imcp 192.168.20.0 0.0.0.255 host 192.168.10.5

SW2 (Config-ext-nacl)# permit ip any any

SW2 (Config-ext-nacl)#interface vlan 20

SW2 (Config-if)# ip access-group CCN in

Some other ACLs were also created to solve some problems but the pattern to create the ACL  is the same as in the above example I took the whole network and used the deny command over it. Screenshots of ACLS working are given below in the verification section.

SOME SCREENSHOTS OF SOLAR PUTTY FOR VERIFICATION PURPOSE:

- I have attached two pics below that shows that switch 1 was used as a client in VTP mode and switch 2 was used as a server.

- I used the command “do sh vlan” in the config mode to get this image:

In the above pic, you can see that I have created VLAN 10 and 20 with the name of DEV AND PRO.

- I used the command “do sh ip interface brie” here to get this image:

Here in the above pic, it is clearly shown that I have created VLAN interface and assigned IP to them which acts as a gateway for the devices.

- I took the screenshots before applying ACL to the project to check whether they can access each other or not. Both the Vlans were able to ping each other and all the 4 devices were able to communicate with each other

PC1

PC2

PC3

PC4

- AFTER APPLYING ACL TO THE PROJECT,

PC1

PC2

PC3

PC4

In the above pics, it is clearly shown that the VLANs were not able to communicate with each other because of the ACL. As we ping the PC of the other VLAN, the output is “communication administratively prohibited”

Below is the screenshot that shows the overall ACLs that were created. Some of them worked and some of them didn’t due to some error. However, overall it worked as you can see the screenshots above that shows the changes after applying the ACL

RESOURCES:

- https://gns3.com/community/featured/how-to-configure-acls-for-3-vlan

- https://learningnetwork.cisco.com/s/question/0D53i00000Kt6za/vlan-access

- https://www.computernetworkingnotes.com/ccna-study-guide/configure-extended-access-control-list-step-by-step-guide.html

- https://www.youtube.com/watch?v=A4Mt1Vgx-fw

- https://www.youtube.com/watch?v=BwEcN_bXLkw&t=174s

- https://www.youtube.com/watch?v=IoM4GeLnm5w

- https://www.youtube.com/watch?v=xmCsnoxaWi4&t=304s

- https://www.youtube.com/watch?v=iY1mtCZN11Q&t=2595s

- https://www.youtube.com/watch?v=ittKxuI4FwY&t=236s

- https://networklessons.com/cisco/ccie-routing-switching/vlan-access-list-vacl